This project demonstrates how to make an MCU expresso IDE's SDK project standalone.

To demonstrate this we use the frdmk22f development board.

First create the SDK example project

We started by creating a  new-> sdk example -> select frdmk22f

Now select next and then expand the demo_apps menu and select the "led_blinky" project  and click next

In the next window you will be prompted to select some project configurations such as floating point settings, compiler selection

And memory configuration ( this is where you would change the memory configuration if you want a non default memory configuration for your project.

Now create the standalone project ( i.e not using the SDK )

New -> Create a new C/C++ project

From this menu select Preinstalled MCUs and select the Generic -M4 and click next

From this menus select "C Project" and click next

In the project name menu type the name you want to give your standalone project and click next.

In this menu you can select the floating point support and the  name of the directory where source is located. You may chose as appropriate for your project but make sure floating point support is similar to the SDK project settings you are going to import to this project.

In this screen you can select the FLASH and RAM allocations for your project to match the flash and RAM locations of the FRDM K22. (We will leave it unchanged for now and change it later in the process).

Importing files and changing project settings

The SDK based blinky project has the following files

The newly created standalone project has the following file structure

Copy folders into standalone project

Copy the CMSIS, board, component, device , drivers, startup, utilities, Debug folders into the standalone project location.

Copy the contents of "SDK projects"/ source  folder into "stand alone project" /src location.

Delete files inside standalone project

Inside  "stand alone project" /src delete the following files inside cr_startup_cm4.c , flashconfig.c, K22Standalone.c

Create a new folder called "stand alone project" /linker

Create a new folder inside "stand alone project" /       called linker.

Now copy the following files inside  "SDK projects"/ Debug to  "stand alone project" /linker

Copy frdmk22f_led_blinky_Debug.ld, frdmk22f_led_blinky_Debug_library.ld, frdmk22f_led_blinky_Debug_memory.ld  into "stand alone project" /linker

Changing standalone project settings

- Add #defines

Open Properties-> C/C++ General -> Paths and Symbols now click the #Symbols

Now copy all the #defines and corresponding values  from  "SDK projects" to "stand alone project" , after copying the list should look like above.

- Add the new source paths

Open Properties-> C/C++ General -> Paths and Symbols now click the Source Location

Add the newly added folders that contain source code, it should look like above

- Add the linker script

Open Properties-> C/C++ Build -> Settings click Tool Settings  tab, now click Managed Linker Script menu under MCU Linker

De select Manage liner script check box.

Add the folder where the linker is located in the Script path box which should be ../linker/

The Linker script box should contain the name of the main linker file , "frdmk22f_led_blinky_Debug.ld"

You should be able to build the project successfully now

The Debug configuration settings should look like this